Supplementary material

Prioritizing CFM Aero-Engine Maintenance-Review Signals from Public Service Difficulty Reports: An FMEA-Informed Robustness Study

# S1. SDR query protocol and interpretation

All keyword searches used Segment = ENGINE (2), Engine Manufacturer containing ‘cfm’, and the listed phrase in Remarks. Other fields were unrestricted unless automatically populated by the AviationDB interface. Counts are query outputs, not unique events, exposure-normalized rates or component failure counts. Observation years are inclusive.

Supplementary Table S1. Query counts, category types and observation periods.

| Signal label | Category type | Keyword | Count | Period |
| --- | --- | --- | --- | --- |
| Stall event | Event-level | stall | 906 | 1986-2024 |
| Fan blade issue | Component/system-level | fan blade | 667 | 2012-2024 |
| Vibration issue | Condition/symptom-level | vibration | 363 | 1986-2024 |
| Compressor issue | Component/system-level | compressor | 297 | 1986-2024 |
| Fuel pump failure | Component/system-level | fuel pump | 239 | 1990-2024 |
| Fuel leak | Condition/symptom-level | fuel leak | 195 | 1993-2024 |
| Sensor fault | Component/system-level | sensor | 188 | 1990-2024 |
| Crack indication | Condition/symptom-level | crack | 167 | 1986-2024 |
| Oil pressure issue | Condition/symptom-level | oil pressure | 114 | 1986-2024 |
| Turbine issue | Component/system-level | turbine | 104 | 1988-2024 |
| Oil leak | Condition/symptom-level | oil leak | 101 | 1998-2024 |
| Oil quantity issue | Condition/symptom-level | oil quantity | 99 | 1987-2024 |
| Engine shutdown | Event-level | engine shutdown | 90 | 1999-2024 |
| Bearing failure | Component/system-level | bearing | 84 | 1986-2024 |
| Fuel pressure issue | Condition/symptom-level | fuel pressure | 1 | 2005 |

# S2. Severity and detection inputs

Higher severity indicates greater consequence; higher detection indicates poorer detectability before manifestation. The values are single-author ordinal screening assignments. They are separated from public query evidence and subjected to bounded perturbation.

Supplementary Table S2. Severity and detection scoring rubric.

| Score | Severity | Detection |
| --- | --- | --- |
| 9–10 | Catastrophic or potentially catastrophic consequences; severe structural or flight-safety impact | Very difficult to detect before manifestation; latent or inspection-limited condition |
| 7–8 | Hazardous or major operational impact; possible shutdown, severe degradation or secondary damage | Detection possible but incomplete; may require special inspection or may progress between checks |
| 5–6 | Moderate operational impact; maintenance significant but generally controllable | Generally detectable through indications, routine inspection or diagnostics |
| 1–4 | Minor to low operational impact | High detectability or early warning likely |

Supplementary Table S3. Category-level severity and detection assignments with rationale.

| Signal label | S | D | Rationale |
| --- | --- | --- | --- |
| Stall event | 9 | 5 | Unstable flow or combustion manifestation with possible surge, thrust loss, shutdown or secondary damage; manifestation is evident but precursors may be incompletely represented in narrative SDR data. |
| Fan blade issue | 10 | 7 | Fan-blade damage can have catastrophic downstream consequences; precursor fatigue or damage may require targeted inspection and may be difficult to identify before manifestation. |
| Vibration issue | 7 | 3 | Abnormal vibration can indicate rotating-system distress but is generally observable through cockpit indications, monitoring systems or maintenance diagnostics. |
| Compressor issue | 8 | 6 | Compressor anomalies may cause performance loss, stall or surge, shutdown or secondary damage; underlying conditions may require diagnostics or inspection. |
| Fuel pump failure | 7 | 5 | Fuel-delivery component failure may impair thrust availability or dispatch reliability; detection may require pressure or flow indications and troubleshooting. |
| Fuel leak | 8 | 4 | Fuel leakage is safety-significant because of flammable-fluid exposure, although many leaks are externally observable or inspection-detectable. |
| Sensor fault | 5 | 3 | Instrumentation or monitoring faults affect system awareness but are often indicated by built-in tests, cockpit messages or maintenance diagnostics. |
| Crack indication | 8 | 8 | Cracking indicates structural or material degradation; growth may be hidden, localized or dependent on special inspection access. |
| Oil pressure issue | 7 | 4 | Lubrication-pressure anomalies can lead to engine damage but are often indicated by warnings or maintenance indications. |
| Turbine issue | 9 | 7 | Hot-section distress may produce severe thermal or mechanical damage; direct inspection access is limited and degradation may progress between checks. |
| Oil leak | 6 | 4 | Lubrication leakage is operationally important but is often visible or identifiable during servicing or inspection. |
| Oil quantity issue | 5 | 3 | Oil-quantity discrepancies are maintenance-significant but are normally detectable through indications, servicing records or routine checks. |
| Engine shutdown | 8 | 5 | An engine shutdown is a serious operational outcome rather than a single causal mechanism; the underlying precursor requires record-level causal coding. |
| Bearing failure | 8 | 7 | Bearing degradation can produce severe secondary damage and internal progression may be difficult to identify without targeted monitoring or inspection. |
| Fuel pressure issue | 6 | 4 | Fuel-pressure anomalies may affect delivery or engine response and are normally moderately detectable through pressure indications and troubleshooting. |

# S3. Primary decision matrix

Window-adjusted count equals query count divided by inclusive searchable observation years. O(window) is the normalized primary occurrence proxy; O(raw) is retained for sensitivity. Baseline(window) = O(window)×S×D.

Supplementary Table S4. Full decision matrix.

| Signal label | Count | Years | Count/year | O(window) | O(raw) | S | D | O×S×D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stall event | 906 | 39 | 23.231 | 0.453 | 1.000 | 9 | 5 | 20.375 |
| Fan blade issue | 667 | 13 | 51.308 | 1.000 | 0.736 | 10 | 7 | 70.000 |
| Vibration issue | 363 | 39 | 9.308 | 0.181 | 0.401 | 7 | 3 | 3.810 |
| Compressor issue | 297 | 39 | 7.615 | 0.148 | 0.328 | 8 | 6 | 7.124 |
| Fuel pump failure | 239 | 35 | 6.829 | 0.133 | 0.264 | 7 | 5 | 4.658 |
| Fuel leak | 195 | 32 | 6.094 | 0.119 | 0.215 | 8 | 4 | 3.801 |
| Sensor fault | 188 | 35 | 5.371 | 0.105 | 0.208 | 5 | 3 | 1.570 |
| Crack indication | 167 | 39 | 4.282 | 0.083 | 0.184 | 8 | 8 | 5.341 |
| Oil pressure issue | 114 | 39 | 2.923 | 0.057 | 0.126 | 7 | 4 | 1.595 |
| Turbine issue | 104 | 37 | 2.811 | 0.055 | 0.115 | 9 | 7 | 3.451 |
| Oil leak | 101 | 27 | 3.741 | 0.073 | 0.111 | 6 | 4 | 1.750 |
| Oil quantity issue | 99 | 38 | 2.605 | 0.051 | 0.109 | 5 | 3 | 0.762 |
| Engine shutdown | 90 | 26 | 3.462 | 0.067 | 0.099 | 8 | 5 | 2.699 |
| Bearing failure | 84 | 39 | 2.154 | 0.042 | 0.093 | 8 | 7 | 2.351 |
| Fuel pressure issue | 1 | 1 | 1.000 | 0.019 | 0.001 | 6 | 4 | 0.468 |

# S4. Weighting and ranking

All criteria are risk-increasing. CRITIC, entropy and equal weights are calculated after column-wise min–max normalization. TOPSIS scores are ranked descending. VIKOR Q values with v = 0.5 are ranked ascending.

Supplementary Table S5. Primary criterion weights.

| Criterion | CRITIC | Entropy | Equal |
| --- | --- | --- | --- |
| Occurrence proxy | 0.359710 | 0.559190 | 0.333333 |
| Severity | 0.240479 | 0.166460 | 0.333333 |
| Detection | 0.399811 | 0.274350 | 0.333333 |

Supplementary Table S6. Full primary ranking results.

| Signal label | Baseline | T-C | V-C | T-E | V-E | T-Q | V-Q |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Fan blade issue | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Crack indication | 4 | 2 | 3 | 3 | 3 | 3 | 3 |
| Turbine issue | 8 | 3 | 5 | 4 | 5 | 4 | 5 |
| Bearing failure | 10 | 4 | 6 | 5 | 7 | 5 | 6 |
| Stall event | 2 | 5 | 2 | 2 | 2 | 2 | 2 |
| Compressor issue | 3 | 6 | 4 | 6 | 4 | 6 | 4 |
| Engine shutdown | 9 | 7 | 8 | 8 | 10 | 7 | 9 |
| Fuel pump failure | 5 | 8 | 7 | 7 | 6 | 9 | 7 |
| Fuel leak | 7 | 9 | 9 | 9 | 9 | 8 | 8 |
| Oil pressure issue | 12 | 10 | 10 | 11 | 11 | 10 | 10 |
| Vibration issue | 6 | 11 | 13 | 10 | 8 | 11 | 12 |
| Oil leak | 11 | 12 | 11 | 12 | 12 | 12 | 11 |
| Fuel pressure issue | 15 | 13 | 12 | 13 | 14 | 13 | 13 |
| Sensor fault | 13 | 14 | 14 | 14 | 13 | 14 | 14 |
| Oil quantity issue | 14 | 15 | 15 | 15 | 15 | 15 | 15 |

Note: T = TOPSIS; V = VIKOR; C = CRITIC; E = entropy; Q = equal weighting.

Supplementary Table S7. VIKOR component values and Q comparisons.

| Signal label | S(CRITIC) | R(CRITIC) | Q(CRITIC) | Q(Entropy) | Q(Equal) |
| --- | --- | --- | --- | --- | --- |
| Fan blade issue | 0.0800 | 0.0800 | 0.0000 | 0.0000 | 0.0000 |
| Stall event | 0.4887 | 0.2399 | 0.4750 | 0.5004 | 0.4592 |
| Crack indication | 0.4324 | 0.3362 | 0.5946 | 0.7520 | 0.6642 |
| Compressor issue | 0.5685 | 0.3124 | 0.6322 | 0.7545 | 0.6831 |
| Turbine issue | 0.4748 | 0.3468 | 0.6344 | 0.7887 | 0.6878 |
| Bearing failure | 0.5276 | 0.3515 | 0.6708 | 0.8178 | 0.7344 |
| Fuel pump failure | 0.7022 | 0.3180 | 0.7146 | 0.8154 | 0.7679 |
| Engine shutdown | 0.6782 | 0.3421 | 0.7390 | 0.8547 | 0.7857 |
| Fuel leak | 0.7393 | 0.3233 | 0.7432 | 0.8395 | 0.7797 |
| Oil pressure issue | 0.8101 | 0.3460 | 0.8176 | 0.9114 | 0.8666 |
| Oil leak | 0.8523 | 0.3401 | 0.8317 | 0.9155 | 0.8896 |
| Fuel pressure issue | 0.8719 | 0.3597 | 0.8732 | 0.9621 | 0.9335 |
| Vibration issue | 0.8444 | 0.3998 | 0.9207 | 0.8324 | 0.9037 |
| Sensor fault | 0.9687 | 0.3998 | 0.9891 | 0.9352 | 0.9901 |
| Oil quantity issue | 0.9885 | 0.3998 | 1.0000 | 0.9823 | 1.0000 |

# S5. Occurrence-specification sensitivity

The secondary specification uses raw keyword-query count normalized by the largest count. It is reported only to show how unequal searchable windows affect ranking.

Supplementary Table S8. Window-adjusted versus raw-count sensitivity.

| Signal label | T(window) | T(raw) | ΔT | V(window) | V(raw) | ΔV |
| --- | --- | --- | --- | --- | --- | --- |
| Fan blade issue | 1 | 1 | 0 | 1 | 1 | 0 |
| Crack indication | 2 | 3 | 1 | 3 | 4 | 1 |
| Turbine issue | 3 | 4 | 1 | 5 | 5 | 0 |
| Bearing failure | 4 | 6 | 2 | 6 | 7 | 1 |
| Stall event | 5 | 2 | -3 | 2 | 2 | 0 |
| Compressor issue | 6 | 5 | -1 | 4 | 3 | -1 |
| Engine shutdown | 7 | 8 | 1 | 8 | 9 | 1 |
| Fuel pump failure | 8 | 7 | -1 | 7 | 6 | -1 |
| Fuel leak | 9 | 9 | 0 | 9 | 8 | -1 |
| Oil pressure issue | 10 | 11 | 1 | 10 | 10 | 0 |
| Vibration issue | 11 | 10 | -1 | 13 | 12 | -1 |
| Oil leak | 12 | 12 | 0 | 11 | 11 | 0 |
| Fuel pressure issue | 13 | 13 | 0 | 12 | 13 | 1 |
| Sensor fault | 14 | 14 | 0 | 14 | 14 | 0 |
| Oil quantity issue | 15 | 15 | 0 | 15 | 15 | 0 |

# S6. Rank correlation

Supplementary Table S9. Spearman rank-correlation matrix.

| Unnamed: 0 | Baseline_rank | TOPSIS_CRITIC_rank | VIKOR_CRITIC_rank | TOPSIS_Entropy_rank | VIKOR_Entropy_rank | TOPSIS_Equal_rank | VIKOR_Equal_rank |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline_rank | 1.000 | 0.757 | 0.821 | 0.850 | 0.939 | 0.807 | 0.861 |
| TOPSIS_CRITIC_rank | 0.757 | 1.000 | 0.946 | 0.971 | 0.907 | 0.975 | 0.946 |
| VIKOR_CRITIC_rank | 0.821 | 0.946 | 1.000 | 0.968 | 0.932 | 0.968 | 0.993 |
| TOPSIS_Entropy_rank | 0.850 | 0.971 | 0.968 | 1.000 | 0.964 | 0.986 | 0.975 |
| VIKOR_Entropy_rank | 0.939 | 0.907 | 0.932 | 0.964 | 1.000 | 0.929 | 0.957 |
| TOPSIS_Equal_rank | 0.807 | 0.975 | 0.968 | 0.986 | 0.929 | 1.000 | 0.971 |
| VIKOR_Equal_rank | 0.861 | 0.946 | 0.993 | 0.975 | 0.957 | 0.971 | 1.000 |

Supplementary Table S10. Kendall rank-correlation matrix.

| Unnamed: 0 | Baseline_rank | TOPSIS_CRITIC_rank | VIKOR_CRITIC_rank | TOPSIS_Entropy_rank | VIKOR_Entropy_rank | TOPSIS_Equal_rank | VIKOR_Equal_rank |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline_rank | 1.000 | 0.600 | 0.676 | 0.695 | 0.829 | 0.638 | 0.714 |
| TOPSIS_CRITIC_rank | 0.600 | 1.000 | 0.848 | 0.905 | 0.771 | 0.924 | 0.848 |
| VIKOR_CRITIC_rank | 0.676 | 0.848 | 1.000 | 0.905 | 0.848 | 0.886 | 0.962 |
| TOPSIS_Entropy_rank | 0.695 | 0.905 | 0.905 | 1.000 | 0.867 | 0.943 | 0.905 |
| VIKOR_Entropy_rank | 0.829 | 0.771 | 0.848 | 0.867 | 1.000 | 0.810 | 0.886 |
| TOPSIS_Equal_rank | 0.638 | 0.924 | 0.886 | 0.943 | 0.810 | 1.000 | 0.886 |
| VIKOR_Equal_rank | 0.714 | 0.848 | 0.962 | 0.905 | 0.886 | 0.886 | 1.000 |

# S7. Perturbation analysis

Severity and detection scores were independently shifted by −1, 0 or +1, clipped to 1–10, over 10,000 simulations. CRITIC weights and TOPSIS scores were recomputed in each simulation. Random seed = 42.

Supplementary Table S11. Perturbation summary for all 15 labels.

| Signal label | Mean | SD | Best | Worst | Top 1 | Top 3 | Top 5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Fan blade issue | 1.000 | 0.010 | 1 | 2 | 99.99% | 100.00% | 100.00% |
| Crack indication | 3.259 | 1.188 | 2 | 10 | 0.00% | 64.25% | 94.90% |
| Stall event | 3.582 | 1.649 | 2 | 10 | 0.00% | 55.11% | 83.72% |
| Turbine issue | 3.717 | 1.314 | 1 | 9 | 0.01% | 48.39% | 89.30% |
| Bearing failure | 4.816 | 1.488 | 2 | 11 | 0.00% | 19.83% | 68.39% |
| Compressor issue | 5.471 | 1.588 | 2 | 11 | 0.00% | 11.88% | 48.55% |
| Engine shutdown | 7.529 | 1.680 | 2 | 13 | 0.00% | 0.47% | 9.87% |
| Fuel pump failure | 8.435 | 1.735 | 2 | 13 | 0.00% | 0.05% | 3.23% |
| Fuel leak | 8.688 | 1.721 | 3 | 15 | 0.00% | 0.02% | 1.98% |
| Oil pressure issue | 10.499 | 1.744 | 5 | 15 | 0.00% | 0.00% | 0.04% |
| Vibration issue | 11.178 | 1.699 | 5 | 15 | 0.00% | 0.00% | 0.02% |
| Oil leak | 11.658 | 1.750 | 6 | 15 | 0.00% | 0.00% | 0.00% |
| Fuel pressure issue | 12.017 | 1.724 | 7 | 15 | 0.00% | 0.00% | 0.00% |
| Sensor fault | 13.952 | 1.085 | 9 | 15 | 0.00% | 0.00% | 0.00% |
| Oil quantity issue | 14.199 | 0.998 | 9 | 15 | 0.00% | 0.00% | 0.00% |

# S8. Reproducibility package

The repository contains data/query_counts.csv and data/scoring.csv as the two immutable analytical inputs; data/decision_matrix.csv is regenerated. Running python src/analysis.py creates the primary and sensitivity results, correlations, perturbation summary and analysis manifest. Running python -m unittest discover -s tests -v executes the integrity tests. Running python src/make_figures.py regenerates all four figures as PNG and 300 dpi TIFF files.